# Changing FXC Pro Marine Settings over NMEA2000

Important considerations:

- The configuration via NMEA 2000 requires the use of the NMEA Command Group Function 126208 (1ED00 hex) PGN.

- The examples mentioned in this document are assuming default values for the AC Input Instance, DC Instance, AC Out Instance, and Inverter Charger Instance. ONLY If you customize those parameters, then you will need to replace the Instance values used in the examples with the custom instance values.

- If you want to see the instance values, then insert a blank USB into the FXC Pro, and wait 30 seconds. Then take the USB out, plug into your computer, and there should be a folder created. Inside the folder, will be a file called “fxcc_user_cfg.yaml”. This file will list all the Instance associations at the top.

- These commands reference field number from this document:

https://xantrex.com/wp-content/uploads/2021/12/976-0422-01-01_Rev-ANMEA2000-PGN-List-for-FXCC_ENG.pdf

## PGN 130913 ( PM – AC Input Configuration Status )

To configure the Breaker Size & AC Lost Level, the following data format is used:

Data: 01 61 FF 01 F8 04 01 A8 00 03 04 08 XX XX 09 XX XX

01 = Command Message
61FF01 = PGN ( 01FF61 = 130913 )
F8 = Priority ( 8 ) 
04 = Number of Pairs of Commanded Parameters To Follow ( 4 )
01 = Field Num of First Param ( Manufacturer Code )
A8 00 = Value of First Param ( 168 decimal, A8 in hex, but the field is 11 bits long, so the value is 00A8h )
03 = Field Num of Second Param ( Industry Group )
04 = Value of Second Param ( 04 = "Marine" )
08 = Field Num of Third Param ( Breaker Size )
XX XX = Value of Third Param ( Scale = 0.1, decoded value must be within 5A – 50A )
09 = Field Num of Fourth Param ( AC Lost Lvl )
XX XX = Value of Fourth Param ( Scale = 0.01, decoded value must be within 85V – 110V )

EXAMPLE:
Let’s say we want to set the AC Breaker size to 30A, and set the AcLostLvl to 104V.

Data: 01 61 FF 01 F8 04 01 A8 00 03 04 08 2C 01 09 A0 28

01 = Command Message
61FF01 = PGN ( 01FF61 )
F8 = Priority ( 8 ) 
04 = Number of Pairs of Commanded Parameters To Follow ( 4 )
01 = Field Num of First Param ( Manufacturer Code )
A8 00 = Value of First Param ( 168 decimal, A8 in hex, but the field is 11 bits long, so the value is 00A8h )
03 = Field Num of Second Param ( Industry Group )
04 = Value of Second Param ( 04 = "Marine" )
08 = Field Num of Third Param ( Breaker Size )
2C 01 = Value of Third Param ( Let’s say we want 30A, then 30A / 0.1 = 300. 300 in hex is 012C )
09 = Field Num of Fourth Param ( AC Lost Lvl )
A0 28 = Value of Fourth Param ( 104 / 0.01 = 10,400 = 28A0h )

Then we send a NMEA Group Function Command using the data listed above. A CAN Log image is shown below displaying command being sent.

1- Full Message

2 - Multi-Packet, ISO Transport Protocol Message

Next, we request the PMAcInputCfgSts PGN using an ISO Request, and the response we receive is:

3 – PMAcInputCfgSts

Decoding Message:

First 11 bits = Manufacture Code = 00010101000 = 168
Next 2 bits = Reserved = 11
Next 3 bits = Industry Group = 100 = 4
Next 8 bits = AC Source Instance = 00000001 = 1
Next 8 bits = Line Total = 00000001 = 1
Next 2 bits = Line = 00 = 0
Next 6 bits = Reserved = 111111
Next 16 bits = Breaker Size = 0000000100101100 = 300, after scaling = 30A
Next 16 bits = AC Lost Level = 0010100010100000 = 10400, after scaling = 104V
For the rest of the fields, we do not support the data, hence the rest of all the data is FFh.

## PGN 127511 ( Inverter Configuration Status )

To control the Inverting, the following data format is used:

Data: 01 17 F2 01 F8 04 01 01 02 02 03 01 04 XX

01 = Command Message
17F201 = PGN ( 01F217 = 127511 )
F8 = Priority ( 8 ) 
04 = Number of Pairs of Commanded Parameters To Follow ( 4 )
01 = Field Num of First Param ( Inverter Instance )
01 = Value of First Param ( 1 )
02 = Field Num of Second Param ( AC Instance )
02 = Value of Second Param ( 02 )
03 = Field Num of Third Param ( DC Instance )
01 = Value of Third Param ( 01 )
04 = Field Num of Fourth Param ( Inverter Enable/Disable )
XX = Value of Fourth Param ( 01 to Enable, 00 to Disable, 11 to leave unchanged )

Example of Enabling Inverting:

4 - Enable Invert ( Multi-Packet & Full Length )

## PGN 65281 ( PM - FXCC Configuration Status )

Field 4 – Buzzer Enable

CAN ID ( hex ): 19ED{FXC CAN Address}{Source Address}
For ex. If the command source address is 0xF8, and FXC CAN node address is 0x00 then the CAN ID is 19ED00F8

Data ( hex ): 01 01 FF  00 08 03 01 A8 00 03 04 04 0X
Valid values for X (decimal, convert to hex to use in data ): {0: 'Off', 1: 'On', 3: 'No Change'}

Field 5 – Inverter Ignition Control

CAN ID ( hex ): 19ED{FXC CAN Address}{Source Address}
For ex. If the command source address is 0xF8, and FXC CAN Node Address is 0x00 then the CAN ID is 19ED00F8

Data ( hex ): 01 01 FF  00 08 03 01 A8 00 03 04 05 0X
Valid values for X (decimal, convert to hex to use in data ): {0: 'Off', 1: 'Lock Out', 2: 'Auto Off'}

Field 6 – Charger Ignition

CAN ID ( hex ): 19ED{FXC CAN Address}{Source Address}
For ex. If the command source address is 0xF8, and FXC CAN Node Address is 0x00 then the CAN ID is 19ED00F8

Data ( hex ): 01 01 FF  00 08 03 01 A8 00 03 04 06 0X
Valid values for X (decimal, convert to hex to use in data ) : {0: 'Off', 1: 'Auto On', 3: 'Data Not Available'}

Field 8 – Hibernate Delay

CAN ID ( hex ): 19ED{FXC CAN Address}{Source Address}
For ex. If the command source address is 0xF8, and FXC CAN Node Address is 0x00 then the CAN ID is 19ED00F8

Data ( hex ): 01 01 FF  00 08 03 01 A8 00 03 04 08 XX
Valid values for XX (decimal, convert to hex to use in data ):  0 – 1500 ( unit is minutes ) ( 0 value means turn the setting OFF

## PGN 65280 ( PM - Device Control Status )

Field 4 – Reset Command

CAN ID ( hex ): 19ED{FXC CAN Address}{Source Address}
For ex. If the command source address is 0xF8, and FXC CAN Node Address is 0x00 then the CAN ID is 19ED00F8

Data ( hex ): 01 01 FF  00 08 03 01 A8 00 03 04 04 0X
Valid values for X (decimal, convert to hex to use in data ): {0: 'Reboot',  4: 'Reset To Factory', 5: 'Reboot To Factory', 6: 'No Change'},

Field 5 – Clear Fault

CAN ID ( hex ): 19ED{FXC CAN Address}{Source Address}
For ex. If the command source address is 0xF8, and FXC CAN Node Address is 0x00 then the CAN ID is 19ED00F8

Data ( hex ): 01 01 FF  00 08 03 01 A8 00 03 04 05 0X
Valid values for X (decimal, convert to hex to use in data ): {0: 'Off', 1: 'On', 3: 'No Change'}

Field 6 – Transfer Mode

CAN ID ( hex ): 19ED{FXC CAN Address}{Source Address}
For ex. If the command source address is 0xF8, and FXC CAN Node Address is 0x00 then the CAN ID is 19ED00F8

Data ( hex ): 01 01 FF  00 08 03 01 A8 00 03 04 06 0X
Valid values for X (decimal, convert to hex to use in data ): {0: 'APL', 1: 'UPS', 3: 'Data Not Available'}

Breaking down the Buzzer command:

01 01 FF  00 08 03 01 A8 00 03 04 04 0X

01 - Command Group Function (write)

01 FF 00 - PGN 65281 (0x00FF01)

08 - Priority Setting 8 = No change

03 - Number of command pairs

01 A8 00 - Field number and value of the manufacturer ID – 168 (0x00a8)

03 04 - Field 3 = Industry Group  - 04 = Marine

04 0X - Field  4 and value for the buzzer

Actual CAN frame:

These are transmitted as Fast Packet in NMEA parlance.  They are sent back to back with the first byte being a sequence number; second byte of the first packet is the length of the whole packet.  So using the example above:

01 01 FF  00 08 03 01 A8 00 03 04 04 0X

Will transmit as two back-to-back packets:

Assuming:

FXC CAN Address = 0x11

Source Address = 0x42

19ED1142:  40 0D 01 01 FF 00 08 03

19ED1142:  41 01 A8 00 03 04 04 0X